Micro Basic Users Guide

I originally wrote Micro Basic in  1980  for  a  University  Computer

Club, in which the members were building their own 8085 based systems.

Dave Dunfield

MICRO BASIC USERS GUIDE                                          Page: 1

1. INTRODUCTION

Micro Basic was implemented with the intention  of  providing  the

maximum amount of features and flexibility, in the minimum amount  of

memory space. It is intended for use on 8080/8085/Z80 based computers

which are too small to afford the use of larger programming  systems.

Currently, the Micro Basic  interpreter,  is  3K  bytes  in  size.  A

machine-language MONITOR and all hardware dependant I/O routines  are

also included, bringing the total rom size to 4K. A minimum of 3K  of

ram is required if any useful programs  are to  be  implemented.  The

interpreter only makes use of memory as it needs it, and  memory  can

be expanded at any time, to allow for larger programs or  more  array

space.  Micro  Basic  is  quite  different  from  most  other   BASIC

interpreters,  in  particular  the  right  to   left   execution   of

expressions, with no operator precedence, and  the  use  of  separate

operators for EQUALS and ASSIGNMENT. These implementation  decisions,

were in part, based on the language APL, which is a favorite language

of the author.

The Micro Basic rom, is assembled to  go  at  location  zero,  and

occupies the range of addresses from 0000-0FFF. The software supports

a 16 line by 64 character 1K  memory  mapped  video  display,  to  be

located in the address range from 1000-13FF. The  system  RAM  should

start at 1400 and may continue on up to FFFF. The keyboard should  be

on a parallel input port,  located  as  port#0,  with  the  keystrobe

(active high) on bit#7, and ASCII data is expected on bits  0-6.  The

keystrobe should remain high for as long as the key  is  pressed. The

serial I/O for the tape is expected to be an INTEL 8251 with the data

port mapped as port#1, and the control port as  port#2.  The  receive

and transmit clocks should be 16x the desired speed.

On the following pages, is a brief description  of  Micro  Basic's

commands and features.

MICRO BASIC USERS GUIDE                                          Page: 2

2. COMMANDS

Operands to commands are as followes:

<e> - Any expression.

<v> - Any variable.

<a> - Any array variable.

<n> - Any numeric expression.

<l> - A line number.

[ ] - Optional operands.

... - Multiple extra operands allowed.

2.1 General commands

The following  commands,  may  be  entered  directly  from  the

keyboard, or executed with a BASIC program.

CLEAR

Clears  all  numeric  and  character  variables,  Delete's  all

arrays, and resets the control stack and data pointer.

DIM a1(<n1>)[,a2(n2)]...

Dimensions integer arrays a1, a2,... makeing  them  n1,  n2,...

elements each. Arrays may be REDIMENSIONED with the DIM statement,

however this allocates new memory for the array, causing  the  old

memory used by the old array to be  made  unusable  (until  'NEW',

'CLEAR' or a 'RUN'  command  is  issued).  Whenever  an  array  is

dimensioned or redimensioned via DIM, it is cleared to zeros.

NOTE: Array space is allocated in memory, starting at the  end  of

the program source. As a result, if a line is inserted  into

the program, or any line is replaced  in  the  program,  any

existing arrays will be deleted.

END

Stops the program. no messages are issued.

EXIT

Exits to the system monitor.

GOTO <line#>

Transfers program execution to the statement at  the  beginning

of line <line#>.

GOTO(<n>),<l1>[,<l2>]...

Transfers program execution to the statement at  the  beginning

of line <l1> if <n>=0, to the beginning of  line  <l2>  if  <n>=1,

etc. results in SYNTAX ERROR if <n> is greater than number of line

numbers given.

MICRO BASIC USERS GUIDE                                          Page: 3

INPUT <v>

Requests a value for <v> from  the  terminal.  Prompts  with  a

question mark "?". If <v> is a character variable, then  any  text

can be input. If <v> is numeric, then value  supplied  must  be  a

number or expression.

INPUT "<text>",<v>

Same as above, but prompts with <text> instead of  "?".  <text>

can be a null string ( INPUT "",<v> would give no prompt ).

NOTE: The value of a character variable can be used in the prompt,

but must be concatinated with <text>. EG: ' INPUT ""+A$,V '.

LET <v>=<e> (default)

Assigns the value of <e> to the variable <v>. If any lines  are

found by the interpreter which do not contain a command, then they

are assumed to be LET.

LIST [<l1>][,<l2>]

Lists the program,if no operands  are  given,  then  lists  the

entire program. If <l1> is given then only that line is listed. If

<l2> is also given, then lists from <l1> to <l2> inclusive.

LOAD

Loads  a  program  from  tape.  Flashes  a  '*'  in  the  upper

right-hand corner of the screen for every record (255 Bytes)  that

is loaded.

NEW

Clears the program, variables, and arrays.

ORDER <line#>

Positions the read pointer to the start of  the  line  <line#>.

This line must begin with a DATA statement, or a  DATA ERROR  will

occur.

PRINT <e>[,<e>][,<e>]...[,]

Prints the expressions on the terminal. Numeric values will  be

printed with  a  preceding  space.  If  a  numeric  expression  is

preceded by a single '(', then the preceding space is not printed.

( EG: PRINT 12,(12 would  display  '  1212'  ).  If  the  list  of

expressions ends with a trailing comma, then no line-feed carriage

return will be  printed,  causing  the  next  PRINT  statement  to

continue at the end of the same line.

MICRO BASIC USERS GUIDE                                          Page: 4

PLOT <x>,<y>[,<e>][,<e>]...[,]

Same as PRINT, except cursor is positioned at column <x>,  line

<y> (from the top lefthand corner of the screen), before the  rest

of  the  operands  are  printed.  'PLOT  <x>,<y>'  with  no  other

operands, will position the cursor with no output.

READ <v>[,<v>]...

Reads the values for variables from data statements.  An  ORDER

statement must be done before the first read  in  a  program,  and

anytime that you have read all the data in a data  block.  A  data

block, is a  collection  of  data  statements  which  are  located

separately, with no other  statements  between  them.  If  a  read

statement does not read all of the data in a given data statement,

then the next read will pick up where the last one left off. If  a

read statement reads beyond the end of a data  statement,  the  it

will advance to the next statement and attempt to read from there.

REM <text>

Comment, the rest of the statement is ignored.

RUN

Clears variables and arrays, then starts the program running. A

running program can be stopped by pressing CONTROL-C.

SAVE

Saves a program on tape.

SIZE

Prints the size of the program in bytes.

STOP

Stops the program, issues message indicating line number  where

it was executing.

USR <n1>[,<n2>][,<v>]

Calls a user supplied machine code routine at address <n1>.  If

<n2> is given, its value will be passed in the H-L register  pair.

If <v> is given, it must  be  a  numeric  variable,  and  will  be

assigned the value of  H-L  after  the  machine  language  routine

returns.

MICRO BASIC USERS GUIDE                                          Page: 5

2.2 Program only commands

The following commands, can only be executed within a program.

DATA <e>[,<e>]...

Defines program DATA, to be read by the  READ  statement,  into

program  variables.  DATA  statements  are  not  executed  by  the

interpreter.

NOTE: Variables can be used in the DATA statements,  but the value

will be evaluated as the value of that variable at the  time

that the DATA statement is read.

FOR <v>=<n1> TO <n2>

Starts a program loop. The variable <v> will be set to <n1>.and

will be incremented by one every time around the  loop.  until  it

value is equal to <n2>. <v> must be a simple numeric variable. See

also 'NEXT' statement.

GOSUB <line#>

Calls a  BASIC  subroutine  at  given  line  number.  (Same  as

goto,but stacks return address.) See also 'RETURN' statement.

GOSUB(<n>),<l1>,<l2>...

Same  as  above,  but  uses  computed  line  number.  See  also

'GOTO(<n>)'.

IF <e> THEN <stmt>

Evaluates  <e>,  If  it  is  true  (non-zero)  then  <stmt>  is

executed. If <stmt> is a number, then assumes GOTO <stmt>.

LIF <e> THEN <stmts>

Long IF, same as IF, except that the entire  remainder  of  the

line is executed only if the expression <e> is true.

NEXT <v>

Closes a program loop. <v> must match the <v> in  the  matching

'FOR' statement.

RETURN

Returns to statement following GOSUB statement.  (Terminates  a

BASIC subroutine)

MICRO BASIC USERS GUIDE                                          Page: 6

3. EXPRESSIONS

Expressions can be either numeric or character.  all  expressions,

are evaluated from right to left, with NO operator precedence (as  in

the language APL). For example, 1+5*5 evaluates to 26, but 5*5+1 will

give the answer 30. Precedence can be forced in  numeric  expressions

with the use of brackets "()". Brackets can be nested to any depth.

3.1 Numeric operators

FORMAT: "X<operator>Y"

+   Addition.

-   Subtraction.

*   Multiplication.

%   Division. (Remainder assigned to special variable "R").

&   Bitwise logical AND of X and Y.

|   Bitwise logical OR of X and Y.

\   Floor. (returns lesser of X and Y).

/   Ceiling. (returns greater of X and Y).

=   Assignment. (X takes value of Y).

==  Equality. (returns 1 if X equals Y, 0 otherwise).

>   Greater than. (returns 1 if X greater than Y, 0 otherwise).

<   Less than. (returns 1 if X less than Y, 0 otherwise).

>=  Greater equals. (Returns 1 if X GE Y, 0 otherwise).

<=  Less equals. (Returns 1 if X LE Y, 0 otherwise).

-=  Not equals. (Returns 1 if X not equal to Y, 0 otherwise).

;   Null operator, returns value of X. (but executes Y).

Especially useful for doing modular arithmetic.

The expression "A=R;B%123" will divide B by 123 (without

changing B), and then assign the remainder to A. (Right

to left execution).

MICRO BASIC USERS GUIDE                                          Page: 7

3.2 Character operators

FORMAT: "X$<operator>Y$"

+   Concatonation. (Y$ appended to X$).

=   Assignment. (X$ takes value of Y$).

==  Equality. (only valid in "IF" and Numeric conversion).

-=  Not equals. (only valid in "IF" and numeric converson)

The following are other operators which perform useful functions:

( ) Brackets, Force operator precedence.

[ ] Braces, Used to index numeric arrays, E.G. "A[10]"

Also can be used to extract a single character

from a character variable. E.G. "A[0]$" returns

the first character in variable "A$".

(Index starts at zero (0) ).

#   Hexidecimal constants. EG. "A=FF#+1" calculates "FF#"

as 255, adds 1 then assigns result (256) to "A".

:   Statement separator, can be used to place multiple

statements on a single program line: E.G: "A=10:PRINT A"

,   Operand separator, separates operands to some commands.

3.3 Numeric conversion

A character expression can be included in a numeric expression,

but must be contained in brackets "()". If the  leftmost  operator

in the character expression is one of "==" or "-=", then a 1 or  0

is returned to the numeric (outside) expression. If  the  leftmost

operator  of  the  character  expression  is  "=",then  the  value

returned is the ASCII value of the  first  character  of  the  OLD

value of the character variable. Otherwise the ASCII value of  the

first character in the  result  of  the  character  expression  is

returned. The ASCII value of a character, is the decimal value  of

it's binary representation. (E.G. " " (blank) is 32).  If  a  null

string ("") is the result of the expression,then the value 255  is

returned (ASCII values for characters can only  range  from  0  to

127). The expression within the brackets does not have to  contain

operators, I.E. " PRINT ("A") " will  print  a  65.  (The  decimal

value of an ASCII "A").

MICRO BASIC USERS GUIDE                                          Page: 8

3.4 Variables

There are 26 simple integer variables  (A-Z).  These  variables

always exist and are cleared to zero when BASIC is entered, when a

NEW command is executed, and when a program is RUN.  Integers  are

positive, with a range of 0 to 65535 (16 bits of data).

There are also up to 26 integer arrays, (A[n] - Z[n]). An array

must be created (via the 'DIM' command) before it  exists.  Arrays

are cleared to zero's when they are created.

Arrays  when  dimensioned,  (DIM  A(n))  have   n+1   elements,

subscripts ranging form 0 to n. Subscripts are not checked by  the

interpreter, therefore, if you type 'DIM A(10),B(10)'  then  A[11]

is the same as B[0].

There are 26  character  variables,  (A$-Z$).  These  variables

always exist and are cleared to null strings ("")  when  BASIC  is

entered, when a NEW is executed,and when a program  is  RUN.  Each

character variable can hold up to 35  characters.  The  individual

characters can be read using braces between the character variable

name, and the dollar sign. (ie. A[0]$ to A[34]$). If an  index  is

greater than 34, a DIMENSION ERROR will result.  If  an  index  is

greater than the number of characters currently in  the  variable,

but less than 35, then  a  null  string  ("")  will  be  returned.

Character variables cannot be assigned values in this manner.

The variable names are all separate, you can have A,  A[n]  and

A$, all in the same program, without interaction between them.

MICRO BASIC USERS GUIDE                                          Page: 9

3.5 Special variables

The simple integer variable 'R' is a special  variable  because

it will be assigned the remainder whenever a divide  operation  is

executed.

The following are special variables, unlike 'R', they cannot be

used as 'normal' variables:

@[n] This variable can only be referenced as an array. When read,

it returns the BYTE value (0-255) of the memory  location at

its index (n). When assigned a value,  that  value  will  be

assigned to the memory location at its index  (n).  (if  the

value assigned is > 255 Then it is divided by 256,  and  the

remainder is used). This is the Same function as 'PEEK'  and

'POKE' in some other BASIC's.

@[n]$ This character variable, can  only  be  referenced  with  an

index. Its index can range from 0 to 255. It will return the

character which has the binary value of its index.  (if  255

is used, it will return a null  string.  This  is  the  same

function as 'CHR$' in some other basics.

? This variable can only be referenced  as  a  simple  integer

variable When read, it returns a random  number  from  0  to

65535. When given a value, it sets the random seed  to  that

value. Random numbers can be generated within limits by  the

use of modular arithmetic. (EG. to generate a random  number

between 0 and 99, and assign it to the variable 'A', use the

command 'A=R;?%100').This is similar to the  'RND'  function

of some other basics.

MICRO BASIC USERS GUIDE                                          Page: 10

4. PROGRAM ENTRY AND EDITING

To enter or replace a line, simply enter it's line number starting

in column one, followed by the text for the new  line.  To  delete  a

line, just enter it's line number, with no following text.

When a line is entered, (and return is pressed), it is copied into

a buffer. Parts or all of this old line can  be  included  in  a  new

line, as it is typed in. When the new line is entered,it then becomes

the old line. A pointer is kept, indicating the current  position  in

the old line. The following functions are available to  perform  this

'editing'.

CTRL-A

Advance: Copy one character from the old line into the  new  line,

and advance the pointer to the next character in the old line.

CTRL-C

Cancel: Cancels the (partially) complete  new  line,  and  restart

from the beginning. (resets old line pointer).

CTRL-D

Delete: Advances the old line pointer by  one  character,deleteing

that character from the old line. the new line is not affected. A '*'

character is printed to indicate this has been done.

CTRL-F

Find: This command requires one extra  character  to  be  entered.

When it is,  the  old  line  is  copied  (from  the  current  pointer

position) into the new line,  up  to  but  NOT  including  the  first

occurance of that character. The pointer is advanced to point to  the

character found. If the character is not found, no action is taken.If

the second character is a carriage return,the remainder  of  the  old

line will be copied into the new line.

CTRL-H

Backup: This backs up one character,deleteing the  last  character

entered, and backs up the old line pointer  in  the  old  line.  This

effectively cancels the effect of the last  character  entered.  (The

DELETE key also invokes this function).

CTRL-M

Carriage Return: enters the new line, causing  it  to  become  the

(new) old line, and passes it to the interpreter, as input.

MICRO BASIC USERS GUIDE                                          Page: 11

CTRL-I

Insert: Toggles insert mode. a "<" is printed when entering insert

mode a ">" is printed when leaving insert mode.  Normally,  when  you

enter text into the new line, the old line pointer  is  advanced,  so

that the characters you are typing, effectivly replace the characters

in the old line. In insert mode, this does not happen, therefore  the

characters you are typing, can be inserted into the old line. (If  it

is later copied into the new line).

The line editor can be used to EDIT program  lines,  When  a  list

command is executed, the last line listed will be made the old  line.

To modify line 50, you would just have to type 'LIST 50'. This  would

display line 50, and would also store it  in  the  old  line  buffer.

Whenever a  program  stops  due  to  an  error,  CTRL-C,  or  a  STOP

statement, the line it stopped on will also be stored in the old line

buffer, ready for editing.

MICRO BASIC USERS GUIDE                                          Page: 12

5. CONTROL CHARACTERS

The following control characters (other than the  ones  recognised

by the line editor) are recognised. All other control characters, are

ignored by the interpreter.

CTRL-C

Will abort any program, terminated with the message "STOP IN  LINE

XXXX" where XXXX is the number of the line containing  the  statement

which would have been executed next.  Will  also  abort  program,  if

entered when responding to an INPUT statement, but will not print the

"STOP" message. Will also abort output from the LIST command.

CTRL-J (line feed)

Will stop terminal output and/or program execution, for as long as

the key is held down. When  released,  output/execution  will  resume

normally.

CTRL-N (SI)

This character, will be ignored when entered  from  the  keyboard,

but when printed, will  cause  the  eighth  bit  to  be  set  in  all

subsequent characters which  are  printed.  Depending  on  the  video

display used,  this  will  cause  REVERSE  VIDEO,  FLASHING,  DIM  or

BRIGHTER output, GRAPHICS CHARACTERS, or  will  have  NO  EFFECT  (If

display ignores extra bit).

CTRL-O (SO)

This character, will be ignored when entered  from  the  keyboard,

but when printed, will cause the eighth bit  to  be  cleared  in  all

characters which are subsequently printed. This will  restore  normal

operation if a special  effect  was  invoked  via  the  CNTRL-N  (SI)

control character above. For example, if  your  video  display  gives

reverse video when the eighth bit  is  set,then  you  could  use  the

following statement to print "REVERSE" in reverse video:

' PRINT @[14]$,"REVERSE",@[15]$ '

CTRL-L (FF)

No effect from keyboard, Clears the screen and  homes  the  cursor

when printed. Eg: ' PRINT @[12]$ '

CTRL-K (VT)

No effect from keyboard, Homes the  cursor  when  printed.  Eg:  '

PRINT @[11]$ '

MICRO BASIC USERS GUIDE                                          Page: 13

6. ERROR MESSAGES

Below is a list of error messages  produced  by  the  interpreter.

Errors occuring in a program, will be followed by  "  IN  LINE  XXXX"

where XXXX is the line on which the error was discovered. All  errors

except '?BAD DATA - RETRY' are fatal,  and  will  stop  an  executing

program.

?SYNTAX ERROR

Results from a statement that is not decodeable. (Does not  follow

syntax) Also results if you attempt to use a  command  in  the  wrong

context. Ie. You use a  command  from  the  keyboard  which  is  only

allowed from within a program.

?NO PROGRAM ERROR

Results from an attempt to RUN or to SAVE an zero line program.

?DIMENSION ERROR

Results from an  attempt  to  index  a  non-array  variable,  from

indexing a character variable  with  a  value  greater  than  34,  or

attempt to PLOT outside the screen.

?DIVIDE BY ZERO ERROR

Results from attempt to divide any value by zero.

?LINE NUMBER ERROR

Results from reference to a program  line  number  that  does  not

exist.

?DATA ERROR

Results from attempt to ORDER to a line which does not start  with

a DATA statement, attempt to READ before you have executed an  ORDER,

reading beyond the end of a DATA BLOCK, or  from  reading  the  wrong

data type (character or numeric) for the operand variable.

?NESTING ERROR

Results from improper nesting of GOSUB/RETURN or FOR/NEXT loops.

?BAD DATA - RETRY

Results from any error  in  a  numeric  expression  typed  as  the

response to an INPUT to  a  numeric  variable.  Does  not  stop,  but

prompts again.

?I/O ERROR

Indicates that a error has occured in the tape system,  Check  for

bad tape.

MICRO BASIC USERS GUIDE                                          Page: 14

7. SOURCE FORMAT

Micro Basic programs are stored  in  memory,  as  variable  length

records, separated by carriage return character (0D hex). The end  of

the program is marked by a  line  starting  with  a  hexidecimal  FF.

Program lines are in the following format:

-------------------------------------------------

|2 bytes|1 byte| variable length section |1 byte|

-------------------------------------------------

\___/   \__/   \_____________________/  \____/

^       ^              ^                 ^_ Carriage Return.

^       ^              ^

^       ^              ^___________________ Program text.

^       ^

^       ^__________________________________ Length of remainder

^                                           of line. (+11 hex)

^

^__________________________________________ Packed decimal line

number (0000-9999).

(FFxx=end of prog.)

MICRO BASIC USERS GUIDE                                          Page: 15

8. EXAMPLE PROGRAMS

The  following  are  some  simple  Micro  Basic  programs,   which

demonstrate many of the features of the language. A good excercise to

gain experience with the interpreter, is to enter and run  them,  and

observe the results.

0010 REM THIS PROGRAM PLAYS THE HIGH/LOW GAME.

0020 PRINT "I WILL PICK A NUMBER BETWEEN 1 AND 100"

0030 PRINT "THEN I WANT YOU TO TRY AND GUESS IT."

0040 PRINT "I WILL TELL YOU IF YOU ARE TOO HIGH, OR TOO LOW"

0050 C=0

0060 N=1+R;?%100

0070 INPUT "WHAT IS YOUR GUESS?",G

0080 C=C+1

0090 LIF G==N THEN PRINT "YOU GUESSED IT IN ",C," GUESSES!":END

0100 IF G>N THEN PRINT "YOU ARE TOO HIGH."

0110 IF G<N THEN PRINT "YOU ARE TOO LOW."

0120 GOTO 70

0010 REM THIS PROGRAM WILL COUNT FROM 1 TO 10, AND DISPLAY

0020 REM THE COUNT, BOTH AS A NUMBER, AND AS A WORD.

0030 ORDER 60 : FOR I=1 TO 10

0040 READ I$ : PRINT I," ",I$

0050 NEXT I

0060 DATA "ONE","TWO","THREE","FOUR","FIVE"

0070 DATA "SIX","SEVEN","EIGHT","NINE","TEN"

0010 REM THIS PROGRAM WILL INPUT N NUMBERS, AND PRINT THEM

0020 REM OUT IN REVERSE ORDER.

0030 INPUT"HOW MANY NUMBERS?",N:DIM A(N):FORI=1TON

0040 PRINT"NUMBER ",I,:INPUT X:A[I]=X:NEXT I

0050 PRINT"NOW HERE THEY ARE BACKWARDS."

0060 FOR I=0 TO N-1:PRINT" ",A[N-I],:NEXTI:PRINT""

0010 REM THIS PROGRAM WILL DISPLAY THE ASCII CHARACTER SET.

0020 FOR I=0 TO 127:PRINT @[I]$:NEXTI:PRINT""

0010 REM THIS PROGRAM WILL DISPLAY THE CONTENTS OF MEMORY

0020 REM FROM 0000 TO 07FF IN DECIMAL.

0030 FOR I=0 TO 7FF# : PRINT @[I],:NEXT I : PRINT ""

0010 REM THIS PROGRAM WILL DISPLAY THE CONTENTS OF MEMORY

0020 REM FROM 0000 TO 07FF IN ASCII.

0030 FOR I=0 TO 7FF# : PRINT @[@[I]]$," ", : NEXT I : PRINT ""

0010 REM THIS PROGRAM WILL INPUT A NUMBER, AND PRINT IT IN HEX.

0020 REM NOTE THE USE OF MOD. ARITHMETIC, AND CHAR. VARIABLE INDEX.

0030 R$="":H$="0123456789ABCDEF":INPUTN

0040 R$=H[R;N=N%16]$+R$:IFN>0THEN40

0050 PRINT R$

MICRO BASIC USERS GUIDE                                          Page: 16

9. THE MICRO BASIC MONITOR

The MICRO BASIC roms include a  machine  language  MONITOR,  which

allows you to read/write memory, load and dump memory to tape, etc.

The monitor has its own set of commands,  which  can  be  entered,

whenever the system is RESET, or  when  ever  BASIC  is  exited  (via

EXIT). Operands to commands are read (and executed) as  soon  as  you

type them in (no carriage return is needed). Hexidecimal  BYTE  (<b>)

values must be entered as TWO characters  (eg  5F),  and  WORD  (<w>)

values must be entered using four characters (eg 00A5).

A summary of commands followes:

B                 Enters the BASIC interpreter,  initializing program,

variables, and arrays.

D                 Downloads Intel hex format through the terminal port.

G <w1>            Go, Begins execution at address <w1>

L                 Loads from tape, indicates record count when  done

(in hex).

M <w1>,<w2>       Displays MEMORY from <w1>, to <w2>.

R                 Reenters BASIC, preserving existing program. Should

not be used when the system is first turned on.

S <w1> XX-<b2>    Substitutes into memory, starting at address <w1> ,

Displays contents as XX, entering <b2> will replace

XX and advance to the next. Entering a  space  will

skip to the next, without changeing, and a carriage-

return will exit to monitor.

T <H> or <F>      Enters terminal mode. Simulates an  ASCII  terminal

Operating in either <Full> or <Half> duplex. Serial

port is tape port.

U <w>             Initalizes the 8251 usart with the value <w>.  See

INTEL 8251 documentation.

W <w1>,<w2>       Writes memory from <w1> to <w2> to tape.

MICRO BASIC USERS GUIDE                                          Page: 17

9.1 MONITOR SUBROUTINES

Address  |Regs.|             Description of

(Hex)|(Dec)|Used |             Subroutine

-----+-----------+-------------------------------------------------------

0037 |0055 |All  | Entry point to monitor.

-------------------------------------------------------------------------

00D6 |0214 |     | Writes memory to tape, DE=start addr., HL=end addr.

-------------------------------------------------------------------------

0162 |0354 |None | Outputs character in A to output device.

-------------------------------------------------------------------------

027E |0638 |None | Checks terminal for CONTROL-C, sets Z if so, LF halts.

-------------------------------------------------------------------------

023F |0575 |  A  | Gets a character for A from input device.

-------------------------------------------------------------------------

02EB |0747 |     | Gets a record from tape, Carry=1 if more data, 0=EOF.

-------------------------------------------------------------------------

034E |0846 |     | Displays message (HL) up to ZERO or carriage return.If

|     |     | carriage return is found, then LF, CR pair is printed.

-------------------------------------------------------------------------

035B |0859 |     | Displays a line-feed, carriage return on the screen.

-------------------------------------------------------------------------

037E |0894 |     | Positions cursor at position in HL in video display.

-------------------------------------------------------------------------

0396 |0918 |     | Starts Cassette deck, and provides delay for speed.

-------------------------------------------------------------------------

03A6 |0934 |     | Stops Cassette deck.

-------------------------------------------------------------------------

|     |     | Displays byte in A on output devices in HEX.

-------------------------------------------------------------------------

|     |     | Displays word in HL on output devices in HEX.

-------------------------------------------------------------------------

|     |     | Gets HEX byte for A from input, CY=0 if failure.

-------------------------------------------------------------------------

|     |     | Gets HEX word for HL from input, CY=0 if failure.

-------------------------------------------------------------------------

0400 |1024 | All | Cold start entry point for  BASIC,  the  program  and

|     |     | variables will be lost, and basic will be started.

-------------------------------------------------------------------------

040D |1037 | All | Warm start entry point for  basic,  the  program  and

|     |     | variables will remain intact.

-------------------------------------------------------------------------

048F |1167 |None | Tests character in A and sets carry flag if it is NOT

|     |     | a valid ASCII numeric digit. ('0' - '9').

-------------------------------------------------------------------------

0499 |1177 | All | Gets a line from  the  terminal.  All  BASIC  editing

|     |     | commands can be used. If CONTROL-C is entered, causes

|     |     | a warm start to BASIC. On exit, D-E is left  pointing

|     |     | to line just entered.

-------------------------------------------------------------------------

MICRO BASIC USERS GUIDE                                          Page: 18

-------------------------------------------------------------------------

067C |1660 | All | Looks up command word (Ending with <space>  or  <cr>)

|     |     | pointed to by D-E, in table pointed to by H-L.  Table

|     |     | format is words in memory, followed by a  blank,  and

|     |     | an address to be returned. End table  with  a  single

|     |     | blank, and default address. On exit, A contains value

|     |     | indicating which word in table was found. (0,1,2,...)

|     |     | and H-L contains associated address.

-------------------------------------------------------------------------

06A8 |1704 |A,B,C| Clears all BASIC variables and arrays, and resets the

|     | H,L | FOR/NEXT, GOSUB/RETURN control stack pointer.

-------------------------------------------------------------------------

06C6 |1734 |A,H,L| As above, but integer and character variables remain.

-------------------------------------------------------------------------

06CC |1740 |A,H,L| As above, but only arrays are cleared, stack remains.

-------------------------------------------------------------------------

077E |1918 |None | Pushes single byte in A on BASIC control stack.

-------------------------------------------------------------------------

0789 |1929 |  A  | Pops single byte to A from BASIC control stack.

-------------------------------------------------------------------------

0792 |1938 |None | Pushes word in D-E on BASIC control stack.

-------------------------------------------------------------------------

079C |1948 | D,E | Pops word to D-E from BASIC control stack.

-------------------------------------------------------------------------

07C9 |1993 |None | Sets carry flag if character in A is not 'A'-'Z'.

-------------------------------------------------------------------------

07D0 |2000 |ABCHL| Stores value in HL into integer variable passed in A.

-------------------------------------------------------------------------

07E1 |2017 |ABCHL| Reads value for HL from integer variable passed in A.

-------------------------------------------------------------------------

07EE |2030 |A,D,E| Advances to next  non-blank  character  in  the  line

|     |     | Pointed to by the D-E register pair.

-------------------------------------------------------------------------

0822 |2082 | All | Evaluates BASIC expression pointed to by D-E,  ending

|     |     | with a carriage return or ':'.  Integer  results  are

|     |     | returned in H-L.

-------------------------------------------------------------------------

08E8 |2280 |ABCHL| Multiply HL by BC, result in HL.

-------------------------------------------------------------------------

091D |2333 | All | Divide HL by BC, result in DE, remainder in HL.

-------------------------------------------------------------------------

0B88 |2952 | All | Displays number in H-L in decimal.

-------------------------------------------------------------------------

0C86 |3206 |ABCHL| Returns in HL the address of the  character  variable

|     |     | passed in A. Character variable format is  35  chars,

|     |     | unused characters at end are padded with $FF.

-------------------------------------------------------------------------

MICRO BASIC USERS GUIDE                                          Page: 19

The MONITOR keeps a byte of data which defines how I/O will  be

handled. It is  located  at  address  $14FF  (5375),  and  can  be

modified by the basic interpreter (or machine  language  program).

It is as followes:

Bits in I/O configuration byte.

+---+---+---+---+---+---+---+---+

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |    (Initial value is 00000011)

+---+---+---+---+---+---+---+---+

|   |   |   |   |   |   |   |

|   |   |   |   |   |   |   +--- 0 = Read from Uart.

|   |   |   |   |   |   |        1 = Read from Keyboard.

|   |   |   |   |   |   +------- 1 = Output to Video Display.

|   |   |   |   |   +----------- 1 = Output to Uart. (8251)

|   |   |   |   +--------------- 1 = Output to User Supplied

|   |   |   |                        Output Device (See Below).

|   |   |   +------------------- 0 = Translate input to upper case.

|   |   |                        1 = Disable UPPER CASE translation.

|   |   +----------------------- 1 = Disable CONTROL-C interrupts.

|   +--------------------------- 1 = Inhibit all output.

+------------------------------- 1 = Set Bit 7 in all characters

output to Video Display.

For example, to prevent a CONTROL-C stop from the keyboard, you

could use the following statement:

' @[5375]=@[5375]|20# '

To reenable CNTRL-C stops, you would use:

' @[5375]=@[5375]&DF# '.

9.2 USER SUPPLIED OUTPUT DEVICE

The system normally supports  two  output  devices,  the  VIDEO

DISPLAY, and the 8251 UART. In addition, it supports  one  logical

output device, to be defined by the user. This is useful for doing

translations before you output a character as well.  To  use  this

feature, you must supply a machine language output routine in  ram

(somewhere outside  the  range  used  by  the  interpreter,  basic

program, and any arrays). Then, store the address of  the  routine

in the USER DEVICE VECTOR, which is located at addresses 14FD  and

14FE (5373-5374), the low order address byte must  go  first.  The

logical user output device is now defined, and can be  enabled  by

setting bit 3 of the  I/O  config.  byte.  NOTE1:  Output  may  be

directed to more than one device at once. NOTE2: If bit#3  of  the

I/O config. byte is set to  1,  before  you  initialize  the  USER

DEVICE VECTOR, Your computer will probably  crash  (And  you  will

lose any unsaved programs).

When the user routine is called, the character to be printed is

passed in the B-REG, and I/O config. byte is passed in recister C.

All of the processor registers may be modified by the subroutine.

MICRO BASIC USERS GUIDE                                          Page: 20

9.3 8251 USART

The 8251 usart, is initialized to 1 stop bit, 7 data  bits,  no

parity, and baud rate of 1/16 times  the  rx  and  tx  clocks.  To

indicate when a tape operation is currently under way, the DSR pin

of the 8251 will be set HIGH (+5v), and  LOW  (0v)  at  all  other

times. A short delay is produced from the time this pin goes high,

and a write to tape operation occures. This allows it to  be  used

as a start/stop control for the tape machine, allowing the tape to

get up to speed.

10. SYSTEM MEMORY MAP

The memory is used by the interpreter as follows:

ADDRESS-RANGE             DESCRIPTION OF CONTENTS.

0000-0FFF   (4k)        Micro Basic Roms.

1000-13FF   (1k)        Memory Mapped Video Display.

1400-14FC   (253 Bytes) Input buffer, Machine stack, Internal storege.

14FD-14FE   (2 Bytes)   User Device Vector.

14FF        (1 Byte)    I/O Configuration byte.

1500-15FF   (256 Bytes) Edit Buffer, Control stack, Internal storage.

1600-19FF   (1k)        Fixed integer and character variables, flags.

1A00-FFFF   (Any size)  Program and Array storage.

When not using the Basic Interpreter (Executing  machine  language

programs from the monitor), Any memory starting at 1500 or higher can

be used without affecting the system. NOTE: Any time  you  enter  the

basic interpreter, anything stored from 1500 to  1CFF  may  be  lost,

Even if you do not enter any program text when in Basic.

MICRO BASIC USERS GUIDE

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

2. COMMANDS                                                            2

2.1 General commands                                                2

2.2 Program only commands                                           5

3. EXPRESSIONS                                                         6

3.1 Numeric operators                                               6

3.2 Character operators                                             7

3.3 Numeric conversion                                              7

3.4 Variables                                                       8

3.5 Special variables                                               9

4. PROGRAM ENTRY AND EDITING                                          10

5. CONTROL CHARACTERS                                                 12

6. ERROR MESSAGES                                                     13

7. SOURCE FORMAT                                                      14

8. EXAMPLE PROGRAMS                                                   15

9. THE MICRO BASIC MONITOR                                            16

9.1 MONITOR SUBROUTINES                                            17

9.2 USER SUPPLIED OUTPUT DEVICE                                    19

9.3 8251 USART                                                     20

10. SYSTEM MEMORY MAP                                                 20