From: Kevin Horton <khorton@tech.iupui.edu>

Subject: Re: Your 8085 stuff!:)

To: Richard Steven Walz <rstevew@armory.com>

Date: Fri, 31 Mar 1995 21:57:12 +6700 (EST)

I didn't know my 8085 stuff was so popular!   I'll e-mail you uuencoded

zipped copies of the assembler, disassembler, and an Intel Hex file

of the monitor.

It's only 9K zipped.  Note that there isn't a .doc file or anything with

it tho.

Monitor requirements:

The monitor fits in an 8K EPROM which must be mapped in at 0000-1FFF (hex).

Also, the monitor uses the top of memory for all functions such as the stack,

temp. register storage, etc.  I usually map an 8K SRAM into C000-FFFF (a

16K segment, of use D000-FFFF depending if I'm using 8 or 16K decoded blocks)

It runs at 2400 baud through the SID/SOD pins if you use a 4Mhz crystal.

Using an 8Mhz crystal gives you 4800 baud! ;-)  It will up/down load

data in intel hex format (uploading doesn't work properly), and report

if there were any problems ('got the data, no transfer errors!', or 'Got the

data, but it's riddled with errors!')  It uses the intel hex sumcheck for

this.  Typing 'menu' gives you a menu of options.  You can enter numbers in

3 diffrent formats- hex, octal, binary.  You can enter hex number two diffrent

ways.

%b1010101  for binary

%o123      for octal

%h34 or 34 for hex

i.e.

LDA 3456

LDA %h3456

LDA %o1234

LDA %b1010101011

The program is not case sensitive.

This monitor is about 2 years old, and as such reflects my bad coding style

back then.  :-\  My newest monitor is almost done, and it's orders of magnitude

better.  I used data compression on all the text (huffman coding), and I

use the bit patterns of the instructions rather than a lookup table for the

mnemonics.  It'll do math on 16-bit quantities as well as logical ops (and

or xor). It has 'memory scan' that will report what kind of memory and

how much is present (ram, rom, not used, filled).  It can dump the memory

contents in various ways- ASCII (without control codes), hex, or both.

You can edit memory using hex or ASCII.  All of these functions work in it

currently.  I'm trying to get the single step stuff working on it.  BTW,

Single Step doesn't work all the time on the version I'm giving you.  Unlike

the current version, this one has well documented source code!  I'm going

to release both the source code and an assembled version Real Soon Now.

I'll announce it in the comp.arch.embedded group when it's done.

Look for a message without any subject line.  It'll have the file in it.

Tell me if it works!

--------------------------------------------------

Oh!

From: Kevin Horton <khorton@tech.iupui.edu>

Subject: Re: 8085 followup

To: Richard Steven Walz <rstevew@armory.com>

Date: Fri, 31 Mar 1995 22:04:35 +6700 (EST)

I just mailed the stuff off.

Oh yeah, I forgot.  I found several undefined opcodes on the OKI version

of the 80C85.  They're cool. There's two word rotates and a few other

conditionals that use the state of the undefined bits in the PSW.

The word rotates are my favourites, tho.  With one byte, you can rotate

the HL left, or the DE right.  (You can't rotate HL right or DE left, tho.

) Also, these rotates do use the carry flag- carry goes into lowest

shift, and the bit on the highest shift gets pushed into the carry.

Also, one of the commands I call 'WHL' (Weird HL) sets the HL to FEEF.

My newest monitor supports these mnemonics.  (Because of this, do I

get to name the mnemonics for these op-codes? ;-)  I looked in the OKI

microprocessors data book and these instructions are termed 'unused'-

truly undocumented.

--

-----------------------------------------------------------------------

| \_ \_  \_\_\_  \_  \_  \_   \_  \_           B5(6) t w g+ s-- r-    |

|  \_\_   \_\_    \_\_    \_   \_\_\_   &h48,&h65,&h6C,&h6C,&h6F,&h21 |

|   \_ \_  \_\_\_  \_      \_   \_  \_   __ khorton@tech.iupui.edu __ |

|     "It's 5:50 AM.  Do you know        \/__Can *YOU* write 8085__\/ |

|     where your stack pointer is?"        \/      assembly?     \/   |

-----------------------------------------------------------------------