*** Z8DIS - DisAssembler for Zilog Z8 microprocessor (pc compatible) ***

Brief Documentation for Z8DIS

-------------------------------

Summary of recognized commands. All commands are 1-2 letters. Required

parameters are enclosed in < >. Optional parameters enclosed in ( ).

All required numeric values MUST be entered in Hex..

rb <filename> (addr) Read binary file into buffer

- Reads a binary file into the 64k z8 program buffer (default load at

address 0000). Optional hex address for start of load.

e.g. rb test.bin 1020

ri <filename> Read Intel File into buffer

- Reads Intel format ascii hex file into z8 program buffer. The first address

of the load operation is displayed at completion.

e.g. ri test.hex

rm <filename> Read Motorola File into buffer

- Same as 'ri' but for motorola format files.

e.g. rm test.hex 1020

u  (pcvalue)  disasm @ pcvalue for 20 lines

- Disassemble z8 program buffer at specified pcvalue. If no pcvalue is

given, disassembly continues from last pcvalue.

e.g. u 1020

uf (pcvalue)  disasm @ pcvalue for 20 lines, also output to file

- Same as 'u' command but output also goes to file. The first time 'uf'

is entered, the user is prompted for a filename. The file is closed at

program termination. Note: the output of the 'u' command is compatible

with the 'Z8CA' cross assembler (C.C.I. bbs).

e.g. uf 1020

d  (hexval)   dumpmem @ hexval for 256 bytes

- Dump z8 program memory in hex/ascii format for 256 bytes starting at

hexval. If no hexval supplied, dump continues from last hexval. Note:

'hexval' is rounded to page (256 byte) address. e.g. ffc1 would be

rounded to ff00.

e.g. d ff00

a  (hexval)   Alter memory starting at hexval

- Allows user to alter z8 program memory on a byte by byte basis, starting

at hexval. The current contents of the memory address is displayed and

can be changed by entering the appropriate hex value. If no change is

desired, simply hit <CR> to advance to the next address. Enter a '.' to

exit this function.

e.g. a 1020

bf <filename> (hexval) Read Basic textfile into mem @ hexval

- This one is for Z8671 (z8 with Basic/Debug) users. Allows the loading

of a Z8 Basic ascii source file directly into z8 program memory.

Performs all the necessary translation to end up with an executable

Basic program. Great for creating an Eprom image for auto-executable

programs. The default load address is 1020 (hex), which is the required

address for auto-execute programs. Load address can be optionally

specified with hexval.

e.g. bf source.bas 1020

Note: Some minor restrictions on the format of the source (basic program)

file are necessary.

1) No blank lines permitted.

2) Sequential Line numbers required.

3) No syntax errors!!

4) No tabs or other invisible characters between the line number

and the start of a program line (spaces ok). No tabs allowed

in program lines

wb <filename> <start addr> <end addr> Write binary file from buffer

- Dumps z8 program buffer to file in binary, delimited by 'start addr' and

'end addr'. Includes the byte specified by 'end addr'.

e.g. wb image.bin 0000 00ff  (dumps 256 bytes to file)

?  Displays the command menu

x  Exits program, closing any open files.

===================== End of Documentation ===============================

Written By: Bob Bush, Compuserve 73105,1332 -or- C.C.I bbs.

Any comments, flame reports, etc. welcome. Any

distribution of this program as part of a commercial product

forbidden, unless permission granted (by me)..