-

- Z8-Basic  Assembly language program development using the Amiga.

-

- Bob Bush 22-Apr-1989

-

Amiga.doc

---------

I happen to use an Amiga, running a 'terminal' program, which provides

access to programming my z8-Basic controller. The terminal program also

allows me to send and receive programs to/from the z8-basic controller

via an ascii file capture & send mode. This provides a convienient method

of z8-basic program storage and retrieval. While this method works well for

the stroage/retrieval of Basic programs, no method exists for the transfer

of assembly language programs.

When I developed the z8 cross-assembler I wanted a quick and easy method of

loading z8 assembly programs using somewhat the same methods I had become

comfortable with for Basic programs. The 'z8.hex' file format (output from

the assembler) allows for this capability.

If you are using a 'terminal' program with the same capabilities I have

mentioned above, you now have a means for transferring z8 assembly

programs. The following documentation will explain how such a transfer

can be accomplished.

Using the z8 format hex file

----------------------------

When you assemble a program and use the '-z' option, a special .hex output

file is created in what I call 'z8' format. This file has the capability to

be loaded directly into z8 program memory provided the user has entered

and executed the 'download.bas' program on their z8-basic controller. The

following steps outline a procedure which works well on my setup.

1) Using your favorite 'terminal' emulator, enter the following program

into your z8-basic controller. This is a very short program (5 lines)

so it shoudn't take long at all..

10 rem z8 program loader

20 "Ready for download:"

30 input b

40 input c:if c>255 b=c:goto 40

50 @b=c:b=b+1:goto 40

2) Execute the program. You should see 'Ready for download' and  a '?'

prompt appear. This means that the program is ready for an assembly

language download.

3) Back to your terminal emulator. Using the same method you use for

transferring Basic progams to the z8-basic controller, send the .hex

file created by the assembler (This happens to be the 'Ascii file

Send' mode on my terminal emulator). You should see a listing of Hex

code appear on the terminal as the program is transferred. The download

is complete when the Basic program stops with an '!427' error (this is

intentional).

4) If everything works as advertised, the assembly code should have been

loaded in z8 memory at the location specified by the 'org' statement

in the assembly source file. You can now test your program by entering

a GO @%xxxx where 'xxxx' is the start address of your program (in hex).

That's all there is to it...

Possible problems:

------------------

Generally, if you have no problems in transferring Basic programs with

your terminal emulator software, you should not experience any problems

transferring assembly code files. Most problems encountered with the

transfer of programs stems from the fact that z8 basic requires some time

at the end of each line to digest whatever was sent over the Rs232 link.

If your  terminal emulator allows for a 'line delay' (this is a settable

delay that occurs after each line is sent from the emulator) try to

increase the 'line delay' parameter. This should solve the problem.

I have tested the shareware terminal emulator 'Access!' and it seems to

work fine at 9600 -or- 19200 baud provided you set the 'line delay' parm

for a short delay between lines.

An integrated solution?

-----------------------

If there is sufficient interest in using the Amiga as a z8 development

tool, I have considered the prospect of developing an integrated

environment which would allow for the development,transfer, and debugging

of z8 assembly (and Basic) language programs. If you are interested in

such a tool please send your comments and or suggestions via Compuserve

email to:

Bob Bush

73105,1332 (compuserve id)

********* end of file **********