-----------------------------------------------------------

Software to PIC12C5xx programming. (c) Jens Dyekjr Madsen

-----------------------------------------------------------

Batch files to program PIC12C508 and PIC12C509:

PROG508  BAT    Normal 12C508 programming utility

PROG508S BAT    Added extra delays

PROG509  BAT    Normal 12C509 programming utility

PROG509S BAT    Added extra delays

READ508  BAT    Reads PIC12C508

READ508S BAT    Added extra delays

READ509  BAT    Reads PIC12C509

READ509S BAT    Added extra delays

Programmer software.

PROG50X  EXE    Standard programming utility

READ50X  EXE    Reading utility

-----------------------------------------------------------

Pascal routines:

SHORTDEL PAS

-----------------------------------------------------------

These routines controls small delays. Between any output

will need to be a small delay, and it controled by the unit.

To allow running from Windows, is interrupts disabled when

timer 2 is used, and the unit wait on timer 2 free before

using it. It allows use on Windows etc.

-----------------------------------------------------------

Pascal routines:

ERRMSG60 PAS

-----------------------------------------------------------

A small unit to include error messages in Turbo Pascal 6.0.

-----------------------------------------------------------

Pascal routines:

INTHEX   PAS

-----------------------------------------------------------

Routines to handle INHX8 files.

-----------------------------------------------------------

Pascal routines:

JDM84IO  PAS

-----------------------------------------------------------

Hardware I/O routines. They are designed to work with all

jdm-programmers. They disable interrupts when sending a

command to ensure that there are no delays at sending

command. It seem to be an error in the PIC's that cause the

PIC to send wrong data if an delay occeur at sending data.

When using this unit, is ensured to keep the timing at

commonication port. The unit also test if a modem is

connected before sending data, to be sure that no modem's

flash is updated. The algorithe may update modem's flash.

-----------------------------------------------------------

Pascal routines:

JDM84PGM PAS

-----------------------------------------------------------

Routines to program / read / erase pic's. The erase routine

has been modify to erase both data and code segment. The

erase routine in microchip specification did not.

-----------------------------------------------------------

Pascal routines:

PICUNIT  PAS

-----------------------------------------------------------

To keep compatibility with old routines.

-----------------------------------------------------------

Pascal utility:

PROG50X  PAS

PROG50XS PAS

READ50X  PAS

-----------------------------------------------------------

A small software utility to program / read PIC12C5xx. Not

used directly, but called by read508.bat and read509.bat,

prog508.bat and prog509.bat.

-----------------------------------------------------------

Pascal routines:

TIMER    PAS

-----------------------------------------------------------

This routine has a timer. The timer works for both windows

and dos software. The idea is to use timer 0, and to use

port[40H] to get high resolution. Timer 0 is programmed to

mode 3 in dos, and to mode 2 in Windows. The timer counts

faster in dos-mode than in windows mode. When in windows

mode, then the timer remainer is the part of ticks. It is

possible to extend the timer delay by using tick counter at

$46C to extend it.

To allow function in both windows and dos, is the timer

reprogrammed to mode 2 in dos. In windows will it not be

programmed. The timer is restored when program abourt, to

mode 3 in dos-mode. Programming to mode 2 makes the timer

to return with remainer as part of ticks in both dos and

windows platforms.

The intension of timer unit is to allow small delays in both

dos and windows.