vhex.doc : 21 Sept 1996

-----------------------

Revision Info

-------------

v1.7 beta 0 - PD release

Now reads and writes in Motorola format, making vhex a translator as

well as viewer.

v1.6 - PD release

More standard CLI

v1.6 beta 0 - beta PD release

Flag-style CLI

(Sorted) Intel hex output option

v1.5 beta 1 - beta PD release

Better QSort.

Fixed a memory leak.

v1.5 beta 0 - beta PD release

First release.

Introduction

------------

VHEX is a utility to read Intel hex files and convert them to a (more)

readable format. I originally wrote it to help locate string segments in

some 8051 code I was working on at the time. It enables a brute-force

approach to code verification, allowing inspection and hand-disassembly

of linked object code.

Currently there are 2 possible output formats, Intel and Readable. Both

are sorted, and the Intel output is terminated, making it a fully usable

hex file, but sorted, which may appeal to some. The readable (if you can

think of a better word for it, tell me) format looks like a hex dump,

complete with a printable ASCII column.

Features

--------

Not many! This is a bonus really, because you know you are seeing what

eventually ends up in the eprom or emulator. It does however thoroughly

check the format and can highlight deficiencies in your hex output if your

eprom programmer or emulator is failing to recognise it.

Because linkers couldn't care less, output is often jumbled, so VHEX sorts

it. It also prints any printable chars. This is useful for locating

copyright messages (the original application) etc. If no output file is

given, it is written to the console.

VHEX reports to DOS via errorlevel. This is useful when batch processing

from a makefile or batchfile. VHEX always does its best to run the course,

and will carry on despite all level 1 errors. Only running out of memory

is a level 2 error and causes immediate exit.

Limitations

-----------

The code is compiled for compact memory model. However, during the quicksort,

an array of pointers to all the records is used. This is subject to the

usual 64K limit, so assuming a four byte pointer, there will be a limit of

16384 records, which is unlikely to be a problem.

It does not recognise Intel-Extended, which appears to be more relevant to

8086-based code, and has segments included.

Long filenames are not supported.

Bugs

----

None known.

Licence

-------

See attached file license.doc.

Mail Me !

---------

If you find this useful, or just if you use it at all, or if you have some

suggestion or complaint, let me know !

==========

Mike Page

mike@mpage.demon.co.uk